Commodity Specific

Food Safety Guidelines

for the

Dry Bulb Onion Supply Chain

1st Edition

December 2010

Special thanks to all of the companies, agencies, trade associations, and individuals who helped in developing the 1st edition of these guidelines.

Acknowledgments:

Primary Sources:

Commodity Specific Food Safety Guidelines for Fresh Tomato Supply Chain, Edition 2, July 2008—United Fresh Produce Association; North American Tomato Trade Working Group (NATTWG)

Model Code for Produce Safety—Association of Food and Drug Officials (AFDO)

1st Edition Editors:

Kay Riley, National Onion Association, Snake River Produce Company

Wayne Mininger, National Onion Association

David Gombas, Ph.D., United Fresh Produce Association

Mike Meyer, Bosgraaf Sales Co.

Doug Stanley, Harris Fresh, Inc.

Greg Bennett, Northwest Onion Co., Inc.

Scott Jensen, Boardman Foods, Inc.

Mike Kirby, Bland Farms, Inc.

Larry Bauman, L & L Ag Production, LLC

Candi Fitch, Idaho-E. Oregon Onion Committee

Robert Komoto, Ontario Produce Company

Marc Asumendi, J.C. Watson Company

Bob Martin, Rio Farms

Additional Contributors and Reviewers:

Nivia Santiago, Gills Onions, LLC

Shannon Kyle, Torrey Farms

John Torrey, Big O Farms, Inc.

Emily Camp, River Point Farms

Bill Dean, River Point Farms

Tim Burnett, Dickinson Frozen Foods

Jami Brock, Taylor Farms

Bob Simerly, McCain Foods USA

Shawn Hartley, Utah Onions

R.T. Sakata, Sakata Farms

Ken Patteson, Diamond Onions, Inc.

Lindsay Dixon, Peri & Sons Farms

User’s Notes:

It is important to remember dry bulb onions, when removed from the ground, have a non-edible surface that protects the onions and should not be washed in this state.  Dry bulb onions are unique. When they have water applied or are washed before the outer skins are removed, it can cause mold and decay to form on the onions.  Onions should be kept at optimum storage temperatures that vary as the season progresses, and they should be kept where there is good air circulation.

These guidelines provide recommended food safety practices that are intended to minimize the microbiological hazards associated with dry bulb onions and fresh-cut/frozen onion products. The intent of drafting this document is to provide currently available information on food safety and handling in a manner consistent with existing applicable regulations, standards, and guidelines.  The information provided is offered in good faith and believed to be reliable, but is made without warranty, express or implied, as to merchantability, fitness for particular purpose, and/or any other matter.  These recommended guidelines were not designed to apply to any specific operation. It is the responsibility of the user of this document to verify that these guidelines are appropriate for its operation.

The publishing trade associations, their members, and contributors do not assume any responsibility for compliance with applicable laws and regulations, and recommend that users consult with their own legal and technical advisors to be sure that their own procedures meet with applicable requirements.  No documented food illness outbreaks have been attributed to dry bulb onions at this time.  These food safety guidelines are an attempt to be proactive and precautionary.

Foreword:

The guidelines presented in this edition represent a current understanding of conditions and controls that should be considered by every company in the onion supply chain for their respective operations.  In some cases, a company may need to consider the guidelines in more than one module.  For example, companies involved in Field Packing should also consider the recommendations in the Open Field Production module, and companies involved in Repacking should also consider the recommendations in the Packinghouse module.

Recently, efforts have been made to more prescriptively define food safety practices for some fresh produce commodities, including the use of quantitative “metrics”.  While that was considered for this edition, the editors recognized that risks and controls are likely to be different between onion sub-commodities and between growing regions and concluded that sufficient science to set metrics is currently lacking.

Therefore, while the editors believe that this edition provides a comprehensive set of considerations, it is left to a future edition to identify a scientifically-based process for setting quantitative acceptance criteria for those considerations.

Commodity Specific Food Safety Guidelines for the Dry Bulb Onion Supply Chain

Table of Contents	Page

Acknowledgements	i

User’s Notes	ii

Foreword		iii

Table of Contents	iv

I.	Introduction	1

II.	Scope and Use of Commodity Specific Food Safety Guidelines for the

Dry Bulb Onion Supply Chain	1

III.	Open Field Production	3

1.	Preventing/Minimizing Risks in the Field - Field Management

2.	Animal Exclusion

3.	Adjacent Land Use

4.	Agricultural Water for Field Use

5.	Hygienic Practices in Onion Fields

6.	Crop Production Practices

7.	Equipment and Containers

8.	Record Keeping

IV.		Harvest Practices	8

1.	Preharvest Assessment

2.	Hygienic Practices in Onion Fields

3.	Equipment and Containers

4.	Debris Removal

5.	Exclusion from Harvest

6.	Culling, Sorting, and Removal of Damaged Onions

7.	Storage

8.	Record Keeping and Traceability

V.		Field Packing	11

1.	Prerequisites for Field Packing Onions

2.	Field Packing Onions

3.	Exclusion from Harvest

4.	Containers for Field Packing Onions

5.	Equipment and Packing Containers in the Field

6.	Transportation of Field Packed Onions

7.	Storage

8.	Traceability, Labeling, and Record Keeping

VI.		Packinghouse	13

1.	Grounds

2.	General Maintenance

3.	Water Supply and Plumbing

4.	Trash and Onion Waste Disposal

5.	Receiving

6.	Packaging Materials

7.	Employee Hygiene, Written Policies, and Employee Training

8.	Handwashing and Toilet Facilities

9.	Handwashing Practices

10.	Health Policies

11.	Other Hygiene Practices

12.	Storage, Packed Product Holding Rooms, and Distribution Facilities

13.	Transportation

14.	Record Keeping, Product Labeling, and Traceability

VII.	Repacking and Other Distribution Operations	19

1.	Prerequisites for Repacking of Onions

2.	Traceability, Lot Identification

3.	Cross-docking and Terminal Markets

VIII.	Fresh-cut and Frozen Processing (Value-added)	20

1.	Receiving

2.	Facility Sanitation

3.	Employee Health and Hygiene

4.	Gloves

5.	Raw Product and Material Storage

6.	Sorting

7.	Cutting

8.	Fresh-cut Onion Washing

9.	Packaging

10.	Storage Rooms and Distribution Facilities

11.	Transportation

12.	Traceability and Labels

13.	Record Keeping

IX.	Foodservice and Retail	26

1.	Purchasing

2.	Receiving - Whole and Fresh-cut Onions

3.	Storage - Whole and Fresh-cut Onions

4.	Facility Sanitation

5.	Employee Health and Hygiene

6.	Preparation within Foodservice/Retail Establishments

7.	Gloves

8.	Storing Cut/Sliced/Diced Onions

9.	Repackaged Dry Bulb Onions

10.	Displaying Cut Onions for the End Consumer

11.		Displaying Dry Bulb Onions for the End Consumer

12.	Traceability and Record Keeping

X.	Appendix	30

I.	Introduction

In 1998, the U.S. Food and Drug Administration (FDA) issued its “Guide to Minimize Microbial Food Safety Hazards for Fresh Fruits and Vegetables.” The practices outlined in this and other documents are collectively known as Good Agricultural Practices or GAPs.  GAPs provide general food safety guidance on critical production steps where food safety might be compromised during the growing, harvesting, transportation, cooling, packing, and storage of fresh produce.  More specifically GAP guidance alerts the entire supply chain, including fruit and vegetable growers, shippers, handler, packers, processors, and buyers to the potential microbiological hazards associated with various aspects of the production chain including: land history, adjacent land use, water quality, worker hygiene, pesticide and fertilizer use, equipment sanitation, and product transportation.  The vast majority of the dry bulb onion industry had adopted GAPs as part of normal production operations.  Indeed the majority of dry bulb onion producers undergo either internal or external third-party GAP audits on a regular basis to monitor and verify adherence to their GAPs programs.  These audits are often shared with customers as verification of the producer’s commitment to food safety and GAPs.  While the produce industry has an admirable record of providing the general public with safe, nutritious fruits and vegetables, it remains committed to continuous improvement with regard to food safety.

In 2004, the FDA published a food safety action plan that specifically requested produce industry leadership in developing the next generation of food safety guidelines for fresh fruits and vegetables. These new commodity-specific guidelines focus on providing guidance that enhances the safe growing, harvesting, processing, distribution, and handling of commodities from the field to the end user.  In the last 12 years, the focus of food safety efforts have been on the farm, drying and distribution points, and value-added processing operations.  Fruit and vegetable processing operations have developed sophisticated food safety programs largely centered on current Good Manufacturing Practices (GMPs) and the principles of Hazard Analysis and Critical Control Point (HACCP) programs.  Food Safety programs for fresh-cut and value added produce have recently been supplemented by FDA’s 2008 “Guide to Minimize Microbial Food Safety Hazards of Fresh-cut Fruits and Vegetables”. As we develop a greater understanding of food safety issues relative to the full spectrum of supply and distribution channels for fruits and vegetables, it has become clear that the next generation of food safety guidelines need to encompass the entire supply chain.

II.	Scope and Use of Commodity Specific Food Safety Guidelines for the Dry Bulb Onion Supply Chain

The scope of this document pertains only to dry bulb onions, and fresh cut/frozen onion products.  This document does not include considerations for products commingled with non-produce ingredients (e.g. salad kits which may contain meat, cheese, and/or dressings), although the onions used in such products should be produced, harvested, and otherwise handled in a manner consistent with the recommendations in this document. The distribution chain for dry bulb onions can be complex in that onions may be sold direct or indirect to the buyer; onions are often subject to repacking for size and/or quality.  As a result, there is no single distribution chain.  The distribution chain may be simple or very complex, with onions being handled by a number of entities prior to being offered for sale to the consumer.  The model distribution chain for the purpose of this document provides an overview of only a few of the many paths a dry bulb onion can take prior to the end user.  It is the intent of this document to cover all significant aspects of the onion supply chain, from production to delivery to the consumer.

Figure 1. General Supply Chain for Dry Bulb Onions

Open Field Production

Harvesting

Field Packing,

Packinghouse, and

Storage

Repacking and other Distribution Operations

Retail/Foodservice

Processing/Preparations

Consumer

Fresh-cut/Frozen (Value Added) Processing

Retail/Foodservice

Outlet

Safe production, packing, processing, distribution, and handling of dry bulb onions and fresh-cut onions depend upon a myriad of factors and the diligent efforts and food safety commitment of all parties throughout the distribution chain.  No single resource document can anticipate every food safety issue or provide answers to all food safety questions.  These guidelines are not intended to replace other food safety programs, but are meant to be used in conjunction with them to address food safety hazards known to affect the onion supply chain. These guidelines focus on minimizing the microbial safety hazards by providing actions, based on the best available science, that have been shown to be effective to reduce, control, or eliminate microbial contamination of onions in the field to fork supply chain. Because of varietal, regional, and operational practice differences, not all of these actions will be applicable to all onion handling operations. However, it is suggested that all companies involved in the dry bulb onion farm to table supply chain consider the recommendations contained within these guidelines in developing their company-specific food safety program.  Every effort to provide food safety education to supply chain partners should be made as well, to ensure that opportunities to prevent contamination are not lost as onions pass from one point of the supply chain to the next. Together with the commitment of each party along the supply chain to review and implement these guidelines, the fresh produce industry is doing its part to provide a consistent, safe supply of produce to the market.

For the purpose of this guidance, the onion supply chain had been divided into seven primary modules:

- open field production,

- harvest practices,

- field packing,

- packinghouse,

- repacking and other distribution operations,

- fresh-cut/frozen processing (value-added), and

- foodservice and retail.

Multiple modules will apply to many users of these guidelines.  Users should not assume that a single module will cover their entire dry bulb operation.

Each of these modules contain key considerations for potential sources of contamination of public health concerns that may occur in the absence of control.

III.	Open Field Production

The development of good agricultural practices for field dry bulb onion production must consider all the elements of the field production system: field site, land use, adjacent land use, agricultural inputs (e.g., irrigation water, fertilizers), workers, and production practices.  Producers are responsible to follow all local, state, and federal labor laws.  Food safety risks can occur from a number of sources; evaluation of these risks, and their management, are essential to proper food safety procedures in the production of dry bulb onions.

1.	Preventing/Minimizing Risks in the Field ― Field Management

Field producers must give consideration to potential sources of microbial, chemical, and physical food safety risks in the selection and management of production sites.

a.	Onion growers should determine previous usage of land if at all possible and 			should assess and mitigate conditions that may pose food safety risk in and 			near production fields.

b.	Conduct an environmental assessment including topography, land history, risk 			of flooding, adjacent land use, and domestic animal and wildlife presence.

i.	Routinely review field environments and maintain records of assessments 				and any corrective actions.

ii.	Consider the potential for flooding to create conditions that may pose a food 			safety risk.  Flooding is the uncontrolled introduction of “flowing or 					overflowing of a field with water outside of the grower’s control, that is 				reasonably likely to contain microorganisms of significant public health 				concern and is reasonably likely to cause adulteration of the edible portion 				of fresh produce in that field”. Additional guidance related to flood events 				can be found in the Appendix.

c.	Onion fields should not be located in any area that can receive direct runoff or 			drainage from a dairy, landfill, other waste, chemical storage, or any other 			likely source of microbial, chemical, or physical food safety hazards.

d.	Steps shall be taken to avoid, prevent, or mitigate run-off into the field from any 			animal operation or other conditions that may pose a food safety risk.

e.	Measures shall be taken to remove or prevent the harvest of onions that have 			been contaminated by runoff from an animal operation.

f.	Procedures used to mitigate risks shall be documented.

2.	Animal Exclusion

a.	Measures shall be taken to exclude domestic animals and livestock from onion 			fields.

b.	Measures shall be taken to minimize the presence of “Animals of Significant 			Risk”, such as cattle, goats, sheep, deer, pigs, and other wild and domestic 			animals.  These measures may include the use of barriers or other deterrents, 			minimizing wildlife attractants and opportunities for harborage, redirecting 			wildlife to non-sensitive areas, and/or by other methods identified by wildlife 			experts.

c.	If animal intrusion is detected, measures shall be taken to remove or prevent the harvest of any contaminated product.

3.	Adjacent Land Use

a.	Assess adjacent land use for activities or conditions that may pose a risk to 			onion safety.  Hazards may include, but not be limited to: livestock, wildlife, 			landfills, sewage treatment, chemical plants, or other conditions that pose a 			food safety risk.

b.	Appropriate measures shall be taken to mitigate any identified food safety 			hazards. These measures may include berms, fences, ditches, buffer zones, or 			other strategies to effectively mitigate any hazards.

4.	Agricultural Water for Field Use

a.	General Requirement:

Water quality shall be adequate for its intended use and shall meet all 				applicable federal and state laws and regulations.

b.	Assessment of Water Source:

i.	The responsible party shall identify, assess the adequacy for its intended 				use, and document all water sources.

ii.	When microbial testing is deemed necessary to verify adequacy of source 				water quality:

1.)	Testing shall be performed and documented using standard indicators of 				fecal pollution, such as E. coli tests. The frequency of testing and point 					of water sampling shall be determined based on the water source, its 					particular history, and the outcome of the risk assessment.

2.)	The results of a microbial analysis of a water source available from a 					public source, such as the local water authority, may serve as 						acceptable documentation in lieu of testing by the grower.

c.	Assessment of Water Distribution System

i.	The responsible party shall prepare a description of the water system in 				use.  This description should be sufficient to facilitate an assessment of the 				risk. This description may use maps, photographs, drawings (hand drawings 			are acceptable), or other means to communicate the location of water 				source(s), permanent fixtures, and the flow of the water system (including 				holding systems, reservoirs, or any water captured for re-use).

ii.	The responsible party shall perform an initial assessment, followed by a 				review (or new assessment) any time there is a change made to the system 				or a situation occurs that could introduce an opportunity to contaminate the 				system.  A water-system assessment shall include an inspection of the 				water system under the control of the responsible party for the purpose of 				identifying conditions that may result in contamination with pathogens of 				concern.

iii.	Water systems intended to convey untreated human or animal waste shall 				be separated from conveyances utilized to deliver agricultural water.

iv.	In the event that the assessment identifies conditions that may result in 				contamination with pathogens of concern, actions shall be taken to correct 				these conditions.

d.	Assessment of Water Use in Crop Production

i.	Growers shall assess the use and quality of water, water application 					methods, application schedules with respect to crop characteristics, and the 				degree of contact with the edible portion of the crop for the purpose of 				identifying conditions that may result in contamination with the pathogens.

ii.	Based on this assessment, growers shall take appropriate action to 					eliminate or minimize the potential for contamination.

e.	Microbial Testing of Agricultural Water

i.	The responsible party shall review the assessments of the water source, 				water distribution system, and water use in regard to the crop 						characteristics, pathogens of concern, proximity to harvest, and other 				relevant factors, and determine the need for microbiological testing of water.

ii.	When microbial testing is deemed necessary, it shall be performed at a 				frequency and sampled at a location based on the assessments, and it shall 			be documented.  When microbial testing is deemed necessary to verify 				adequacy of source of water quality, testing shall be performed as described 			in the Model Code for Produce Safety by the Association of Food and Drug 				Officials (AFDO). Section IV B 2 b) (1) of the AFDO code reads: Testing 				shall be performed and documented using standard indicators of fecal 				pollution, such as generic E. coli tests. The frequency of testing and point of 				water sampling shall be determined based on the water source, its particular 			history, and the outcome of the risk assessment.

5.	Hygienic Practices in Onion Fields

Ensure that production crews, visitors, and other field personnel are aware of food safety risk reduction principles and they agree to adhere to the firm’s practices and policies.

a.	Written Policies and Employee Training

i.	Operations shall develop and implement written GAP and Employee 					Hygiene	Practices.

ii.	All employees shall receive mandatory safe product handling and personal 				hygiene education at time of hire, with periodic reinforcements, at least 				seasonally.

iii.	Training sessions shall be documented with records of topics covered, date, 			names, and acceptable verification of those in attendance.

iv.	Routine oversight and periodic self-audits shall be used to verify and 					document compliance with worker hygiene and sanitation policies and 				practices.

b.	Cleanliness/Sanitation

i.	Sanitary facilities shall be provided for all field workers and visitors during 				planting, harvesting, or other field activities.  Toilet facilities shall be 					provided with minimum of one per twenty employees and be readily 					accessible, located not more than ¼ (0.25) mile of all employees.

ii.	Toilet facilities shall be designed, located, operated, and serviced in a 				manner 	that does not pose a source of contamination of the field.

iii.	Toilet facilities shall have appropriate hand washing stations, including 				collection of gray water.

iv.	Toilet facilities shall be maintained in a clean and sanitary condition and 				properly stocked with soap. Water for hand washing that meets the 					microbial standard for potable water, single-use towels, toilet paper, etc., 				and a written record of cleaning shall be kept.

v.	Restroom cleaning equipment shall be labeled and segregated so as not to 				pose a risk of contamination.

vi.	Policies shall require hand washing with soap and water at appropriate 				times such as before starting work, after breaks, using the restrooms, 				sneezing, or coughing.

c.	Health

i.	Employees, visitors, and other field personnel with symptoms of diarrhea, 				fever, vomiting, or other potentially infectious illnesses shall be restricted 				from working with or in the vicinity of onions or onion-contact surfaces.

ii.	Employees, visitors, and other field personnel with open sores, cuts, burns, 				boils, etc., shall report to a supervisor before working or entering the field.  				The supervisor shall determine if the employee will be allowed to work with, 				or in the vicinity of, onions or onion-contact surfaces.

d.	Hygiene

i.	Employees, visitors, and other field personnel with symptoms of diarrhea, 				fever, vomiting, or other potentially infectious illnesses shall be restricted 				from working with, or in the vicinity of, onions or onion-contact surfaces.

ii.	There shall be a written policy prohibiting eating, drinking, chewing gum, 				and using tobacco in fields except in clearly designated areas.

iii.	Drinking water shall be provided with either fountains or single-use 					containers.  Drinking water containers shall be handled in a manner that 				prevents them from becoming sources of contamination.

iv.	Other good food handling techniques shall be developed as appropriate to 				the specific operation to prevent cross contamination.

6.	Crop Production Practices

Assess risk of all production inputs to reduce contamination risk.

a.	Chemical Fertilizers

i.	Follow manufacturer’s instructions for usage and storage.

b.	Fertilizers Containing Manures, Composts, or Biosolids

i.	Only properly treated manures and biosolids are allowed for use in onion 				fields.

ii.	If treated manures or biosolids are used, records of composition, dates of 				treatment, methods utilized, application dates and any test results, or 					process verification data demonstrating compliance with microbial standards 			must be documented.

c.	Pesticides (Crop Protection Treatments)

i.	Pesticide chemicals used must comply with all requirements of EPA 					registration and any federal, state, or local regulations.

ii.	Pesticides must be appropriately registered for such use and must be used 				in accordance with label directions.  Pesticides used shall be documented.

iii.	Pesticides shall be applied by trained, licensed, or certified pesticide 					personnel, as required by regulation.

iv.	Pesticides for foliar application shall be mixed with water that meets Model 				Code for Produce Safety (AFDO) standards (page 4, Sec. 4).

d.	Chemicals Used on Product

i.	Chemicals used on product that are not registered pesticides may be 					permitted for food-contact use if allowed under regulations of the U.S. Food 				and Drug Administration (FDA).

7.	Equipments and Containers

a.	Onion storage containers need to be clean and free from foreign material and in 		good repair.

b.	Reusable containers and food-contact equipment and utensils shall be 				constructed of materials that can be easily cleaned.

c.	Clean and maintain containers, bins, food-contact equipment, and utensils as 			needed to remove sand, grit, dirt, and other residue.

d.	Establish routine cleaning procedures and maintain these standard operating 			procedures in writing.

e. 	Maintain all equipment and surfaces in such a way as to minimize 				contamination of, and injury to, onions.

f.	All containers shall be marked for their intended use (trash, etc.)

8.	Record Keeping

Appropriate record keeping provides evidence of operating conditions and 	practices and facilitates periodic review and evaluation of those practices.

a.	Records documenting adherence to these practices, such as those addressing 			environmental assessments, employee training, water usage, pest control, crop 		production practices, and any needed corrective actions, for the operations 			must be maintained and producible in a reasonable amount of time.

b.	The source of all agricultural inputs used in the production of the crop (e.g., 			seeds, transplants, fertilizers, pesticides) shall be recorded.

c.	Records shall be retained for at least two years, or as required by regulation.

IV.	Harvest Practices

Onions for harvest shall have been produced according to GAPs and the recommendations described in the prior section on Open Field Production.

1.	Preharvest Assessment

A preharvest assessment provides a last opportunity to evaluate any safety risks 	from the field that may impact the potential for the onions to be contaminated. The field man, ranch manager, or other responsible person shall ensure that an assessment is performed as close as practical prior to the beginning of harvest, for example, not more than 7 days prior to the beginning of harvest.

a.	Conduct an environmental assessment including topography, land history, 			adjacent land use, and domestic and wildlife presence.

i.	Review field environments and records of assessments and corrective 				actions performed during the current production period.

b.	Onion fields should not be located in any area that can receive runoff or 				drainage from an animal operation or any other source of contamination.

c.	Domestic animals and livestock have been excluded from onions fields.

d.	Wildlife presence has been minimized.

e.	If animal intrusion is detected, measures shall be taken to remove or prevent 			the harvest of any potentially contaminated product.

f.	Run-off from any animal operation has been prevented.

g.	The source of water for irrigation for each crop has been documented and 			criteria have been met.

h.	Procedures used to identify risks and mitigate those risks have been 				documented, followed, and are reviewed.

i.	If onions are harvested at multiple times, fields should be assessed sufficiently 			to assure that new risk factors have not emerged.

2.	Hygienic Practices in Onion Fields

Ensure that harvest contractors and crews have been trained in food safety risk reduction principles and that they agree to adhere to the firm’s practices.

a.	Written Policies and Employee Training

i.	Operations shall develop and implement written GAP and Employee 					Hygiene	Practices.

ii.	Training sessions shall be documented, with records kept of topics covered, 			date, names, and acceptable verification of those in attendance.

iii.	Periodic (e.g. daily, weekly, monthly, quarterly, as appropriate) self audits 				shall be used to verify and document compliance with worker hygiene and 				sanitation policies and practices.

b.	Sanitary Facilities

i.	Toilet Facilities and Hand-Washing Stations

1.)	All toilet facilities and hand washing stations shall be:

a.)	Kept clean, well supplied with toilet paper, water, soap, and paper 						towels, and shall be accessible and properly located;

b.)	Directly accessible for servicing, serviced and cleaned on a 							schedule sufficient to ensure stability for use; and

c.)	Located as to minimize the potential risk for field and produce 							contamination.

ii.	Water used for hand washing shall meet the microbial standards for drinking 			water prescribed in the Code of Federal Regulations (CFR): 40 CFR Part 				141.63.

c.	Sewage Disposal

i.	Sewage and septic systems shall be maintained in a manner to prevent 				contamination of growing fields or produce with pathogens, and in 					compliance with local laws and regulations.

1.)	Portable toilet facilities shall be serviced in a location and manner that 					does not pose a risk of contamination of growing fields or produce with 					pathogens.

2.)	The responsible party shall have a plan for immediate control and 						treatment of any effluent in the event of leakage or a spill. Leakages or 					spills shall be managed and disposed of in accordance with applicable 					federal, state, and local laws and regulations, and in a manner that 					prevents or minimizes contamination of growing fields or produce with 					pathogens.

d.	Health

i.	Worker health policies shall restrict employees with symptoms of diarrhea, 				fever, vomiting, or other potentially infectious illnesses from working with, or 				in the vicinity of, onions or onion-contact surfaces.

ii.	Employees with open sores, cuts, burns, boils, etc., shall report to a 					supervisor before working. The supervisor shall determine if it is safe to 				allow the employee to work with, or in the vicinity of, onions or onion-contact 			surfaces.

e.	Hygiene

i.	Employees shall have designated areas for eating, drinking, smoking, 				breaks, personal effect, etc.

ii.	There shall be a written policy prohibiting eating, drinking, chewing gum, 				and using tobacco in fields except in clearly designated areas.

iii.	Drinking water shall be provided with either fountains or single-use 					containers.  Drinking water containers shall be handled in a manner that 				prevents them from becoming sources of contamination.

iv.	Other good food handling techniques shall be developed as appropriate to 				the	specific operation to prevent cross contamination.

f.	Harvest crews are trained to recognize and report any food safety risks or 			hazards observed during the harvest operation.

3.	Equipment and Containers

a.	Onion storage containers need to be clean and free of foreign material and in 			good repair.

b.	Reusable containers and food-contact equipment and utensils shall be 				constructed of impervious materials that can be cleaned.

c.	Any containers used to hold onions that are received back from a packing 			house must be checked for cleanliness prior to use.

d.	Clean and maintain harvest containers, bins, food-contact equipment, and 			utensils as needed, to remove sand, grit, dirt, and other residue.

e.	Establish routine cleaning procedures and maintain those standard operating 			procedures in writing.

f.	Maintain all equipment and surfaces in such a way as to minimize 				contamination of and injury to onions.

g.	Records shall be maintained of cleaning procedures and their implementation.

4.	Debris Removal

Dirt and chaff should be removed from onions to the degree practical in the field, 	in a manner that does not pose a risk of contamination.

5.	Exclusion from Harvest

a.	Onions contacted by fecal material shall not be harvested.

b.	If animal intrusion is detected, measures shall be taken to remove or prevent 			the harvest of any potentially contaminated product.

c.	Damaged, soft, or decayed onions should be excluded, to the degree 				possible.

6.	Culling, Sorting, and Removal of Damaged Onions

Damaged, soft, or decayed onions should be removed, to the degree possible.

7.	Storage

a.	Any area or facility used to collect or store onions must be maintained in a 			clean and sanitary manner.

b.	In bulk storage, new disposable foot covers must be worm when inspecting 			onions.

8.	Record Keeping and Traceability

Record keeping provides evidence of reviews and evaluations to document those practices.  Records shall also be kept to assure traceability of harvested onions.

a.	Records documenting adherence to these practices, such as those addressing 			preharvest assessments and employee training for the operation 					must be maintained and producible in a reasonable amount of time.

b.	Traceability practices shall be utilized to ensure that all onions are traceable to 			their origin, at least one step forward and one step back.

c.	Records shall be retained for at least two years, or as required by regulation.

V.	Field Packing

Field packing of onions includes any practices to grade, sort, size, clean, pack, or palletize onions in the field into containers for commerce.  Field packed onions are not intended to be transferred to a packing house for further handling.  Care must be taken to ensure that practices and conditions do not contribute to contamination.

1.	Prerequisites for Field Packing Onions

Packing of onions in the field must meet all Good Agricultural Practices (GAPs) included in this document in Section III, Open Field Production, including management, site, and adjacent land use, water use, and hygienic practices, production practices, harvesting procedures, and record keeping, in addition to the requirements further detailed in this Section of Field Packing.

2.	Field Packing Onions

Employees packing onions in the field shall be supervised in order to ensure the safety of the product. Hygienic practices for field packing employees shall be followed and verified by supervisors. These hygienic practices shall include handwashing and sanitizing.

a.	Culling

Packing onions in the field generally occurs with mature onions so extra care to 		cull and remove any damaged onions shall occur.

b.	Hygienic Procedures

Minimum legal requirements for field sanitization facilities and procedures are 			prescribed in the Occupational Safety and Health Act, 29 CFR, Part 1928.110.

c.	A written procedure for hygienic practices for field packed operations and 			records showing compliance must be available.

d.	Documentation of employee training on hygienic procedures for the field 				packing of onions shall be retained and available.

3.	Exclusion from Harvest

a.	Onions contacted by any fecal material shall not be harvested.

b.	If animal intrusion is detected, measures shall be taken to remove or prevent 			the harvest of any potentially contaminated product.

c.	Damaged, soft, or decayed onions should be excluded to the degree possible.

4.	Containers for Field Packing Onions

All containers shall be stored in a manner to prevent contamination. Special attention shall be given to contamination risks from rodents, birds, and other pests.

a.	All packaging material is inspected upon arrival and stored in a clean manner.

b.	Containers used for field packing may not be stored in the field unless 				protected from potential contamination.

c.	Picking and packing containers shall be distinguishable from those serving 			other purposes.

d.	Reuse of single-use containers (e.g., corrugated) for field use of packing is 			prohibited.

e.	Reusable containers, such as reusable plastic containers (RPCs) and totes, 			shall be clean. Ensure that labels are accurate prior to reusing for packing.

f.	Containers shall be protected from direct contact with the ground.

g.	Containers shall be properly labeled with information sufficient for traceability, 			including identification of the firm packing the onions.

5.	Equipment and Packing Containers in the Field

a.	Any surface that touches onions postharvest should be considered a food-			contact surface and should be treated or handled in a manner so as not to be a 			source of contamination of the onions.

b.	Harvest containers, food-contact surfaces, and utensils shall be cleaned and 			maintained as needed to remove sand, grit, dirt, and other residue.

6.	Transportation of Field Packed Onions

a.	Transportation vehicles should be sufficiently clean so as not to be a source of 			contamination.

b.	Inspect transportation vehicles for cleanliness, odors, visible dirt, and debris 			before loading.  If needed, the vehicle shall be cleaned prior to use.

c.	If non-dedicated vehicles are used for transportation, verify from records that 			vehicles have been sufficiently cleaned if prior loads may be a potential source 			of contamination.

7.	Storage

Any area used to collect and store onions packed in the field must be maintained in a clean and sanitary manner.

8.	Traceability, Labeling, and Record Keeping

All onions shall be traceable at least one step forward and one step back.  This shall include appropriate labeling in each case.

a.	Documentation of field packed onions shall include sufficient information about 			the harvest (i.e., field location and history, grower, personnel/crew involved in 			the harvesting and packing) as well as customer receiving the product to allow 			for the appropriate tracing of product.

b.	Containers shall be accurately labeled with the commodity name, field packer, 			firm name, and information sufficient to allow for identification of grower, ranch 			and field location, harvest crew, and date of harvest/field pack.

c.	Labels that are inaccurate shall be removed prior to packing.

d.	A documented recall program, including a traceability system to track onions 			forward to customers, shall be developed and tested at least annually.  A 			record of this test shall be maintained and be available.

e.	Traceability records shall be readily available.

f.	All records recommended in this section shall be maintained for at least two 			years and be readily available.

VI.	Packinghouse

A well designed and managed packinghouse and food safety program can greatly reduce the risk of chemical, physical, and microbial contamination, but the risk can never be totally eliminated.  Poor or inconsistent food safety practices can greatly increase this risk. Sanitary conditions and proper food safety practices are critical to product safety.

The needs of each packinghouse may vary due to location, environment, the volume of onions handled, the type of onions handled, local regulations, and many other variables, but the overall goal of any effective packinghouse food safety program is to minimize risk of contamination.  There are multiple strategies for effectively dealing with individual hazards.

The general requirements for the packing of dry bulb onions are that facilities shall meet the requirements for packinghouse and grounds, processing, packing, holding, and retailing of foods, equipment and utensils, sanitary facilities and controls, sanitary operations, and processes and controls as provided for under 21 CFR part 110 or its equivalent, as appropriate to each facility.  This shall extend to all aspects of the packinghouse, including storage and packed product rooms.

1.	Grounds

a.	The grounds about a packinghouse under the control of the operator shall be 			kept in a condition that will protect against contamination of onions.  The 				methods for adequate maintenance of grounds include, but are not limited to:

i.	Properly storing equipment, removing litter and waste, and cutting weeds 				or grass within the immediate vicinity of the plant buildings or structures 				that may constitute an attractant, breeding place, or harborage for pests.

ii.	Maintaining roads, yards, and parking lots so that they do not constitute a 				source of contamination in areas where onions are exposed.

iii.	Adequately draining areas that may contribute contamination to food by 				seepage, foot-borne filth, or providing a breeding place for pests.

iv.	Operating systems for waste treatment and disposal in an adequate 					manner so that they do not constitute a source of contamination in areas 				where onions are exposed.

b.	If the packinghouse grounds are bordered by grounds not under the 				operator’s control, and not maintained in the manner described in paragraph 			(a) (i) though (iii) of this section, care shall be exercised in the packinghouse 			by inspection, extermination, or other means to exclude pests, dirt, and filth 			that may be a source of contamination.

c.	It is recommended that the land adjacent to the packinghouse should not be 			a significant source of contamination. Hazards may include, but not be 				limited to livestock, wildlife, landfills, chemical plants, etc.

d.	Appropriate measures shall be taken to minimize any food safety hazards 			from surrounding land use or environment.  These measures may include 			berms, fences, ditches, buffer zones, or other strategies to effectively 				mitigate any hazards. Records shall be kept of the measures used.

2.	General Maintenance

a. 	Buildings, fixtures, and other physical facilities of the packinghouse shall be 			maintained in a clean and sanitary condition, and shall be kept in repair 				sufficient to prevent food from becoming adulterated. Cleaning and sanitizing 			of utensils and equipment shall be conducted in a manner that protects 				against contamination of food, food-contact surfaces, or packaging materials.

b.	Establish Sanitation Standards Operating Procedures (SSOPs) related to the 			general cleaning and sanitation of the facility, including maintenance of dump 			tanks, bump pads, brush rollers, sponge rollers, and other equipment.  While 			a cleaning schedule is part of the SSOPs, the volume of onions handled may 			require frequent attention to cleaning.

c.	Cleaning compounds, sanitizers, pesticides, and all other chemicals shall be 			labeled, handled, and stored in a manner that does not pose a risk of 				contamination to food, food-contact surfaces, or food-packaging materials.  			Food-grade and non food-grade chemicals shall be kept separate in order to 			minimize the risk of accidentally substituting one for the other.  These 				products shall be kept separate in accordance with manufacturers’ labels, 			instructions, and all federal, state, and local regulations shall be followed.

d.	Pest Control

Rodents, birds, amphibians (e.g., tree frogs) reptiles, and other facility pests.

i.	A written and implemented pest control program shall be in place to 					protect the packinghouse from pests.

ii.	The use of insecticides or rodenticides is permitted only under precautions 				and restrictions that will protect against the contamination of food, food-				contact surfaces, and food-packaging materials. Generally, only non-toxic 				traps and pest control devices are used inside the packinghouse.

iii.	No domestic animals or other animals are permitted in areas where onions 				are being handled.

e.	Sanitation of food-contact surfaces

i.	All food-contact surfaces, including utensils and food-contact surfaces of 				equipment, shall be cleaned and sanitized in keeping with an established, 				documented SSOP to protect against contamination of food.

ii.	Non food-contact surfaces shall be cleaned and sanitized in accordance 				to the facility’s SSOP, or more frequently if necessary to protect onions 				from contamination.

iii.	Single-service articles (such as utensils intended for one-time use, paper 				cups, or paper towels) should be stored in appropriate containers and 				shall be handled, dispensed, used, and disposed of in a manner that 					protects against contamination of food or food-contact surfaces.

iv.	Sanitizing products shall be registered for their intended use, and cleaning 				and sanitizing products used according to manufacturers’ labeling 					instructions.

f.	Cleaned and sanitized portable equipment with food-contact surfaces and 			utensils should be stored in a location and manner that protects food-contact 			surfaces from contamination.

3.	Water Supply and Plumbing

a.	The water supply shall be sufficient for the operations intended and shall be 			derived from an adequate source.  Any water that contacts food or food-				contact surfaces, intended or unintended, shall meet the microbial standards 			as set forth by the Environmental Protection Agency (EPA) for drinking 				water.

b.	Running water shall be available at suitable temperature and volume where it 			is needed for packing, cleaning, sanitation, and employee hygiene.

c.	Plumbing

Plumbing shall be of adequate size and design and adequately installed and 			maintained to:

i.	Supply sufficient quantities of water to required locations throughout the 				packinghouse.

ii.	Properly convey sewage and liquid disposable waste from the 						packinghouse in a manner that does not pose a risk of contamination to 				food, water supplies, equipment, and utensils, or create an unsanitary 				condition.

iii.	Provide adequate floor drainage in all areas where floors are subject to 				flooding-type cleaning, or where normal operations release, or discharge 				water, or other liquid waste on the floor.

iv.	Protect against backflow from or cross-connection between piping systems 				that discharge wastewater or sewage and piping systems that carry water 				for food or food manufacturing.  Appropriate backflow prevention devices 				(e.g., air gaps, backflow valves) shall be used to protect water quality at the 				source and during distribution and use.

d.	Sewage disposal

Sewage shall be properly disposed into appropriate sewer, septic, or alternative 		systems that do not pose a risk of contamination.

4.	Trash and Onion Waste Disposal

Trash and onion waste shall be handled, stored, and disposed in a manner that minimizes odors, minimizes the potential for attracting and harboring pests, and minimizes the risk of contamination of onions, food and non food-contact surfaces, and water supplies.

5.	Receiving

a.	Ensure onions are from suppliers following GAPs or other recognized, similar 			food safety requirements, and these guidelines.

b.	Establish a written procedure for inspecting, accepting, or rejecting incoming 			loads.

c.	Ensure that incoming documentation provides sufficient information to facilitate 			traceability to the source.

d.	Records of incoming inspections shall be maintained.

6.	Packaging Materials

a.	Packaging materials shall be inspected upon arrival. The goal is to ensure that 			packaging material is free from contamination upon arrival and that materials 			are stored in a means as to prevent contamination.

b.	The packinghouse shall minimize the risk of contamination by adopting written 			plans that address each of the following issues:

i.	All packaging material is inspected upon arrival and stored in a clean 					manner.

ii.	Pallets used to keep finished product off the floor are visually clean.

iii.	Bins, trays, and pallets are maintained in clean operational condition 					according to SSOPs.

iv.	Bins, trays, and pallets are stored in a secure, clean location.

v.	Finished product containers are distinguishable from those serving other 				purposes.

vi.	There is no evidence of rodent, bird, or insect infestations in the storage 				locations.

7.	Employee Hygiene, Written Policies, and Employee Training

a.	Facilities shall develop and implement written GMPs and Employee Hygiene 			Practices.

b.	All employees shall receive mandatory safe product handling and personal 			hygiene education at time of hire and at least annually.

c.	Training sessions shall be documented, with records kept of topics covered, 			date, names, and acceptable verification of those in attendance.

d.	Periodic (e.g., daily, weekly, monthly, quarterly, or as appropriate) self audits 			shall be used to verify and document compliance with worker hygiene and 			sanitation policies and practices.

8.	Handwashing and Toilet Facilities

a.	Restrooms shall be available to all personnel (at least one toilet for every 20 			employees) and located in proximity to food handling areas, but not so close 			that they could be a source of contamination. Restrooms should not open 			directly into food-handling areas.  Restrooms that do open directly into food-			handling areas should be equipped with self-closing mechanisms or have 			maze-type entrance/exit.

b.	Toilet facilities shall be maintained in a clean and sanitary condition and 				adequately stocked with soap, water for handwashing that meets the microbial 			standards for potable water (including hot water where available), single-use 			towels, toilet paper, etc.

c.	A written record of cleaning shall be kept.

d.	Restroom cleaning equipment shall be labeled and segregated so as not to 			pose a risk of contamination.

e.	Handwashing signs shall be posted in restrooms. Signs should be multilingual 			or pictorial, as appropriate to workforce.

f.	Other handwashing facilities.

Handwashing facilities shall be adequate in number and location, and be 			furnished with running water at a suitable temperature.  Compliance with this 			requirement may be accomplished by providing:

i.	Handwashing and, where appropriate, hand-sanitizing facilities at each 				location where good sanitary practices require their use.

ii.	Soap and water for handwashing that meets the microbial standard for 				potable water (including hot water where available).

iii.	Single-use towels or air drying devices.

iv.	Handwashing signs shall be posted at all stations.  Signs should be 					multilingual or pictorial, as appropriate for work force.

v.	Refuse receptacles that are constructed and maintained in a manner that 				protects against contamination of food.

vi.	Sanitizers may not be used in lieu of proper handwashing.

vii.	Provisions shall be in place for capture, disposal, or drainage of gray water 				in a manner that prevents contamination of the environment.

9.	Handwashing Practices

Written polices shall require handwashing with soap and water at the appropriate time, such as before starting work, after breaks, visiting locker rooms, using the 	restrooms, sneezing, coughing, and touching any unsanitary surface or material.

10.	Health Policies

a.	Worker health policies shall restrict employees with symptoms of diarrhea, 			fever, vomiting, or other potentially infectious illnesses from working with, or in 			the vicinity of, onions or onion-contact surfaces.

b.	Employees with open sores, cuts, burns, boils, etc., shall report to a supervisor 			before working.  The supervisor shall determine if it is safe for the employee to 			work with, or in the vicinity of, onions or onion-contact surfaces.

c.	Establish and communicate a clear policy that prohibits workers who report or 			are observed to have diarrhea, or symptoms of illness, from activities that may 			contact onions or onion-contact surfaces.

11.	Other Hygienic Practices

a. 	Employees shall have designated areas for eating, drinking, smoking, breaks, 			personal effects, etc.

b.	There shall be written policy prohibiting eating, drinking, chewing gum, and 			using tobacco in fields or facilities except in clearly designated areas.

c.	Drinking water shall be provided with either fountains or single use containers.  			Drinking water containers shall be handled in a manner that prevents them from 		becoming sources of contamination.

d.	There shall be written policy restricting jewelry and other accessories that 			could become a physical contamination in the final packed product in the 			workplace.

e.	Employees shall wear clean and suitable outer garments.  Consider, as 				appropriate to the operation, hair restraints, plastic aprons and sleeves, 				restricting nail polish or false nails, and empty pockets above the waist.

f.	Outer garments and gloves shall be changed after cleaning drains, restrooms, 			or other activities that may result in contamination.

g.	Other good food handling techniques shall be developed as appropriate to the 			specific operation to prevent cross contamination.

12.	Storage, Packed Product Holding Rooms, and Distribution Facilities

a.	Storage, packed product holding rooms, and distribution facilities shall be kept 			clean and sanitary with debris minimized. All walls, floors, ceilings, and other 			surfaces shall be systemically and periodically cleaned.

b.	Packed product shall be palletized, or contain a barrier, to avoid direct contact 			with the floor.

c.	 A perimeter between pallets and walls shall be maintained to facilitate visual 			inspection of pest control and sanitation in packed product rooms, packing 			supply rooms, and distribution facilities.

d.	Product on hold, or rejected, shall be clearly identified and segregated from 			other product.

e.	There shall be no storage of trash or waste in the storage, packed product or 			packing supply rooms, and distribution facilities.

13.	Transportation

a.	Transportation vehicles should be sufficiently clean so as not to be a source of 			contamination.

b.	Inspect transportation vehicles for cleanliness, odors, visible dirt, and 				debris before loading.  If needed, the vehicle shall be cleaned, or cleaned and 			sanitized by a documented procedure prior to loading.

c.	If non-dedicated vehicles are used for transportation, they should be checked 			for cleanliness.  Should there be any potential food safety risks, such as from 			raw animal proteins, garbage, or other refuse, the vehicle shall be cleaned by a 			documented procedure prior to use.

14.	Record Keeping, Product Labeling, and Traceability

All levels of the onion supply chain shall maintain adequate traceability to a minimum of one step forward (immediate next recipient) and one step back (immediate previous supplier).

a.	Documentation maintained by the packinghouse shall include sufficient 				information about the source (i.e., production location, lot identification, 				personnel/crew involved with harvesting, and transporters) as well as the 			customer receiving the product to allow for the appropriate tracing of product, 			21 CFR part 1, subpart J.

b.	The packer shall have established procedures to ensure that traceability 				information about the source is retained with product as it moves through 			packinghouse processes to shipping.

c.	Corrugated containers shall be new and accurately labeled with commodity 			name, packinghouse firm name, and lot identification sufficient to allow for 			accurate traceability.  Reusable containers (e.g., RPCs) and totes, shall be 			assessed for their potential to become sources of contamination and cleaned 			sufficiently to prevent contamination of future loads. Ensure that labels are 			accurate prior to using for packing.

d.	A documented recall program, including a traceability system to track onions 			forward to customers, shall be developed and tested at least annually.  A 			record of this test shall be kept on file.

e.	All records recommended in this section shall be maintained for at least two 			years and be readily available.

VII.	Repacking and Other Distribution Operations

Everyone in the supply chain that handles onions, including repackers, terminal markets, and other facilities, has a responsibility to ensure and maintain the safety and traceability of the product.

1.	Prerequisites for Repacking of Onions

Repacking of onions must meet all requirements included in this document in Section VI - Packinghouse, including, receiving, water supply and plumbing, trash and onion waste removal, general maintenances, packing material requirements, employee hygiene, written policies and employee training, handwashing and toilet facilities, handwashing practices, health policies, other hygienic practices, storage, product labeling/traceability, and transportation, in addition to the requirements further detailed in this Section on repacking.

2.	Traceability, Lot Identification

All levels of the onions supply chain shall maintain adequate traceability to a minimum of one step forward (immediate next recipient) and one step back (immediate previous suppler).  In addition to requirements described in Section VII-Packinghouse, repacking operations shall:

a.	Establish procedures to maintain lot identity of onions throughout the repacking 			process.

i.	Documentation maintained by the repacker for each lot received shall 				include sufficient information about the source (i.e. production location, 				supplier identification, lot identification) as well as the customer receiving 				the product to allow for the appropriate tracing of product.

ii.	Ensure that the information is retained with product as it moves through the 				packinghouse processes to shipping.

iii.	It is preferred that incoming lots are not mixed/commingled during 					repacking. However, if incoming lots are mixed/commingled, then 					documentation shall be maintained to identify all included sources.

iv.	Traceability records shall be readily available.

v.	Effectiveness of these procedures shall be tested at least annually.  A 				record of this test shall be kept on file.

b.	If onion lots are not mixed/commingled, then onions may be repacked into their 			original boxes.  When original containers of a packinghouse supplier are to be 			reused, and the onions are removed and resorted, and returned to that clean 			and sanitary container, the repacker must label the container as being 				repacked. The commodity, repacker name, and lot identification must be 				provided.

c.	If onion lots are commingled, then containers should be accurately labeled 			with the repacker’s information and lot identification that maintains the integrity 			of traceability information to the included sources. In the event of a recall, all 			lots in the commingled lot are affected.

d.	Used boxes and sacks may only be used as secondary shipping containers, 			provided that the original identification information on the box has been 				obliterated or otherwise made clear that it is no longer accurate.  Used boxes 			may only be used as primary containers for mixed/commingled lots if they pose 			no food safety risk and the original identification information on the box is still 			accurate to the original source of all of the onions in the box.

3.	Cross-docking and Terminal Markets

Onion handling at facilities that primarily redistribute onions, whether or not they repack, sort, or otherwise change the contents in the container, are also required to follow the recommendations in these guidelines as appropriate for their specific operation.

VIII.	Fresh-cut and Frozen Processing (Value-added)

Processing fresh-cut and frozen produce requires strict adherence to sanitary practices in a controlled environment.  Processing fresh produce into fresh-cut products increases the risk of bacterial contamination by breaking the natural exterior barrier of the produce.

Fresh-cut processing operations are required to comply with GMPs identified in 21 CFR part 110 and any prevailing state or local regulations, and should consider FDA recommendations identified in FDA’s 2008 Guide to Minimize Microbial Food Safety Hazards of Fresh-cut Fruits and Vegetables.

Frozen onions are preserved by freezing in such a way that the range of temperature to maximum crystallization is passed quickly.  The freezing process shall not be regarded as complete until the product temperature has reached 0°F (-18°C) or lower at the thermal center, after the thermal stabilization.  Produce will remain in its frozen state 0° to -20°F (-18° to -28°C) to maintain shelf-life.

1.	Receiving

a.	Establish food safety procedures and requirements.

b.	Ensure onions suppliers follow GAPs, GMPs, as appropriate, or other certified 			food safety audit programs.

c. 	Establish a written procedure for inspecting, accepting, or rejecting incoming 			loads.

d.	Require documentation that provides sufficient information to ensure 				traceability to the source.

e.	Records of incoming inspections shall be maintained as required by regulation.

2.	Facility Sanitation

Sanitation refers to all hygienic practices designed to maintain a clean environment for food production, storage, and preparation. Comprehensive sanitation programs, with trained sanitation personnel, reduce the risk of product microbial contamination.  Improper use of chemicals may lead to inadequately cleaned equipment or chemical contamination of equipment.  A well implemented pest control program will maintain a sanitary environment for food production by reducing the risk of rodent, insect, or bird infestations, which could lead to product contamination.

a.	Raw, processed, and finished produce segregation shall be addressed by using 		physical barriers.

b.	A documented sanitation program shall be in place that meets regulatory 			requirements and ensures the cleanliness of product handling equipment and 			facility, including storage and processing rooms.

c.	Maintain cleaning frequencies: include peripherals (e.g., walls, ceilings, light 			fixtures, cooling units, etc.) and support areas like lunchrooms, hallways, 			restrooms, and office areas.

d.	Chemicals shall be registered with the EPA or be referenced in the CFR and 			used in accordance with label instructions for time, temperature, concentration, 			and application.

e.	Facilities should maintain Material Safety Data Sheets (MSDS) for all 				chemicals.

f.	A program review shall be implemented that monitors and documents 				adequacy and compliance in the sanitation program.

g.	The results of the verification program shall be documented and monitored to 			identify areas of improvement.

h.	A program (e.g., color coding) shall be in place to readily identify and segregate 		food-contact vs. non-food contact equipment and utensils used in the sanitation 		program.

i.	Hands shall be cleaned and sanitized prior to entering processing areas.

j.	Product and packaging material shall be protected or removed during cleaning 			and sanitizing operations to reduce the potential for cross contamination.

k.	Sanitation personnel shall not spray floors or drains with high-pressure hoses 			(resulting aerosol may contaminate product or product-contact surfaces).

l.	Sanitation personnel shall follow written SSOP’s which specify all steps 				required to clean properly each piece of equipment, adequate chemical 				concentrations, contact time, and utensils needed.

m.	Sanitation personnel shall not place product contact equipment on the floor.

n.	Facilities shall properly identify and segregate equipment used to clean drains 			and floors and shall not use equipment aids with wooden handles.

o.	The effectiveness of the sanitation program shall be verified by analysis of food-contact surfaces using methods such as adenosine triphosphate (ATP) bioluminescence testing, which is a rapid, simple, and reliable test to monitor surface contamination. Environmental swabs that target hygiene indicator organisms such as Coliforms and/or Aerobic Plate Count should also be used to verify the effectiveness of the program to eliminate or minimize bacterial contamination.

p.	A preventive maintenance program shall be in place that identifies and prevents 		opportunities for contamination of onions or onion-contact surfaces with 				pathogens, chemical, or physical hazards of public health concern.

q.	Facilities shall develop and implement a written pest-control program to include 			a licensed pest-control technician, adequate monitoring frequencies, and pest-			control devices to control the infiltration of rodents and insect 					monitoring/control.  Pesticides shall be EPA approved for the methods, target 			pests, and locations where they are used.

3.	Employee Health and Hygiene

a.	Facilities shall develop and implement written GMP and Employee Hygiene 			Practices, with mandatory training for all employees at time of hire or at least 			annually.

b.	Worker health policies shall restrict employees with symptoms of diarrhea, 			fever, vomiting, or other potentially infectious illnesses from working.

c.	Employees with open sores, cuts, burns, boils, etc., shall report to a supervisor 			before working.  The supervisor shall determine if the employee will be allowed 			to work.

d.	Establish and communicate a clear policy that prohibits workers who report or 			are observed to have diarrhea, or symptoms of illness, from activities that may 			contact onions or onion-contact surfaces.

e.	Written policies shall require hand washing with soap and water at the 				appropriate time such as before starting work, after breaks, visiting the local 			locker rooms, using the restrooms, sneezing, coughing, touching any 				unsanitary surface or material, or anytime hands become soiled.

f.	Policies shall require employees to wear clean outer garments, gloves, and 			hairnets.

g.	If required, plastic aprons and sleeves must be worn.

h.	Written procedures shall be developed to define conditions when outer 				garments and gloves shall be changed, such as after cleaning drains, 				restrooms, or other similar areas.

i.	A policy should be established to determine procedure on how to handle crying, 		wiping tears, and runny nose as a reaction from the onion vapors released 			when onions are peeled or cut.

4.	Gloves

The following applies to all operators who handle onions in processing facilities.

a.	Disposable Gloves

i.	The use of single-use disposable gloves for hand contact with onions is 				recommended.

ii.	Hands shall be washed before putting on gloves.

iii.	Hand sanitizers may be used, but not as a substitute for proper washing of 				hands.

iv.	Disposable gloves must be changed after any process involving handling of 				materials other than onions or when gloves become torn, soiled, or 					otherwise contaminated.

b.	Reusable Gloves

i.	Hands shall be washed before putting on gloves.

ii.	Hands shall be washed and sanitized after donning.

iii.	Reusable gloves must be washed and sanitized after any process involving 				handling of materials other than onions.

iv.	Reusable gloves must be replaced when the gloves have become torn, 				soiled, or otherwise contaminated.

5.	Raw Product and Material Storage

a.	Storage container as well as storage facilities shall be clean.

b.	Product and product containers shall be stored in a manner that minimizes the 			potential for contamination.

c.	Even though whole onions can be maintained at room temperature for a 				considerable period of time without affecting quality, better results are achieved 		the sooner that onions are cooled down to refrigeration temperatures for 				processing.

d.	Storage temperatures should also be maintained from 32° to 41°F to ensure 			required finished product temperatures are achieved.

e.	Whole onions should be pre-cooled prior to processing.

6.	Sorting

It is important to remove damaged or decayed raw material and use gentle handling practices to reduce the risk of contamination.

a.	Secondary containers used for packing sorted onions shall be maintained in 			clean and sanitary condition.

b.	Onions that are decayed should be disposed of properly.

c.	The sorting process shall be performed in a manner ensuring that further 			damage to the onion is minimized.

d.	Lot identity shall be maintained throughout the sorting process.

e.	Preventive measures shall be implemented to remove foreign/extraneous 			materials.

7.	Cutting

Cutting blade sharpness and damage should be monitored regularly.  Improperly maintained blades can result in damaged and bruised tissue, which can make the product more susceptible to support microbial growth.

a. 	Peeling and cutting equipment should be of appropriate materials and 				constructed using sanitary design to ensure ease of cleaning and sanitation.

b.	Monitor raw material and finished product temperature to ensure maximum 			shelf life.

c.	Raw material and finished product temperature should be protected from 			contamination, metal inclusion, and other foreign products during processing.

d.	Blade condition relating to sharpness and damage should be monitored 				regularly. Improperly maintained blades can result in damaged and bruised 			tissue which can reduce the shelf life of the product.

e.	Utensils used like knives, scoops, and shovels, must be color coded to 				distinguish them from utensils used for raw material.  They should be kept in 			sanitary condition.

f.	Non-conforming raw materials or finished products must be tagged and 				segregated to avoid contamination.

8.	Fresh-cut Onion Washing

Properly used antimicrobial chemicals help prevent pathogens from contaminating onions or cross-contaminating other onions or onion-contact surfaces.

a.	The source water used in washing shall meet the microbiological requirements 			of drinking water.

b.	The wash water antimicrobial shall be monitored at a frequency sufficient to 			assure it is continuously at levels sufficient to prevent cross-contamination with 			pathogens of public health concern.

c.	Wash water temperature shall be monitored to assure finished products do not 			exceed refrigerated temperatures (<41°F).

9.	Packaging

a.	An effective packaging control system shall be maintained to prevent the use 			of contaminated, damaged, or defective cartons, trays, and totes in order to 			prevent microbial contamination of the fresh-cut onions during packing 				operations.

b.	Packaging material coming into direct contact with the fresh-cut onions shall be 			appropriately identified, including source traceability.

c.	Packaging containers and cartons shall be used for their intended purpose 			only.

d.	Packaging materials shall be stored in a manner to protect them from 				contamination, such as away from pests, dirt, cleaning chemicals, and water 			condensation from overhead equipment and structures.

e.	Primary and secondary finished fresh-cut onion product shall be labeled with 			recommended storage instructions (e.g., “Keep Refrigerated”) and storage 			temperature to inform all persons handling the product of the recommended 			storage conditions.

f.	Primary and secondary packaging shall be coded to ensure traceability.

10.	Storage Rooms and Distribution Facilities

a.	Finished products shall be stored at refrigerated temperatures not to exceed 			41°F.

b.	Finished frozen onion products shall be stored at temperatures not to exceed 			0°F.

c.	Storage rooms and distribution facilities shall be kept clean and sanitary, with 			debris minimized. All walls, floors, ceilings, and other surfaces shall be 				systematically and periodically cleaned and sanitized to avoid the build-up of 			mold or other potential contaminants.

d.	Product shall be palletized to avoid direct contact with the floor.

e.	A perimeter between pallets and walls shall be maintained to facilitate visual 			inspection of pest control and sanitation.

f.	Product on hold, or rejected, shall be clearly identified and segregated from 			other product.

g.	There shall be no storage of trash or waste in storage rooms.

11.	Transportation

Finished products transported in sanitary and refrigerated coolers and vehicles reduce the risk for physical, chemical, and microbial contamination.

a.	Finished products shall be transported at refrigerated temperatures not to 			exceed 41°F.

b.	Finished frozen onion products shall be transported at temperatures not to 			exceed 0°F.

c.	Finished products shall be transported in pre-cooled vehicles equipped with a 			temperature monitoring device.

d.	Transportation vehicles should be sufficiently clean so as not to be a source of 			contamination.

e.	Inspect transportation vehicles for cleanliness, odors, visible dirt, and debris 			before loading.  If needed, the vehicle shall be cleaned, or cleaned and 				sanitized, by a documented procedure prior to loading.

f.	If non-dedicated vehicles are used for transportation, they should be checked 			for cleanliness.  Should there be any potential food safety risks, such as from 			raw animal proteins, garbage, or other refuse, then the vehicle shall be cleaned 		by a documented procedure prior to use.

12.	Traceability and Labels

All levels of the onion supply chain shall maintain adequate traceability to a minimum of one step forward (immediate next recipient) and one step back (immediate previous supplier).

a.	Documentation maintained by the processor shall include sufficient information 			about the source (e.g., production location, packer/repacker, lot identification, 			as appropriate to the source of onions) as well as the customer receiving the 			product to allow for the appropriate tracing of product.

b.	The processor shall have established procedures to ensure that traceability 			information about the source is retained with product as it moves through the 			processes to shipping.

c.	Primary and secondary containers shall be accurately labeled with commodity, 			name, processor firm name or identification code, and lot identification sufficient 		to allow for accurate traceability.

d.	Traceability records shall be readily available.

e.	A document recall program, including a traceability system to track onions 			forward to customers, shall be developed and tested at least annually.  A 			record of this test shall be kept on file.

13.	Record Keeping

Food processors are required to keep records on file to verify processes.

a.	All processing, receiving, and shipping records shall be maintained on file as 			required by regulation.

b.	A document control program should be established to ensure customer 				confidentiality of specification and proprietary documents.

c.	Records to be maintained shall include:

i.	Sanitation records

ii.	Pest control records

iii.	Maintenance records

iv.	Facility inspection records

v.	Employee training records

vi.	Incoming inspection records

vii.	Customer complaint records

viii.	Incoming water quality records

ix.	Water treatment and monitoring records

x.	Equipment and calibration records

xi.	Temperature control records

xii.	Finished product inspection records

xiii.	Microbiological records (environment, product), if required by food safety 				plan.

xiv.	Distribution records

IX.	Foodservice and Retail

1.	Purchasing

Ensure onions are from suppliers following GAPs and/or GMPs, as appropriate, or other recognized, similar food safety requirements, and these guidelines. Practices can be verified through documented self-inspections, audits done by qualified government or private sector food safety auditors, and/or other appropriate mechanisms of assurance.

2.	Receiving - Whole and Fresh-cut Onions

a.	Establish written procedures for inspecting, accepting, or rejecting incoming 			loads.  Procedures should include the condition of transportation vehicles as 			well as incoming product requirements.

b.	Ensure that incoming documentation provides sufficient information to facilitate 			traceability to the immediate prior supplier.

c.	Records of incoming inspections shall be maintained.

d.	Cut onions (i.e., sliced, diced, or chopped) shall be received at <41°F, and 			requires continuous temperature control during transport.

e.	Cut onions >41°F at receipt may be cause for rejection.

3.	Storage - Whole and Fresh-cut Onions

a.	Dry bulb onions shall be maintained at the temperature recommended for the 			variety and the time of season.

b.	Dry bulb onions should not be washed.

c.	Cut onions shall be maintained at <41°F, in accordance with recommendations 			in the current edition of the Food Code, or appropriate state and local 				regulations.

d.	Onions shall be raised off the floor and stored in a manner to prevent cross 			contamination from raw food products, chemical, and unsanitary conditions.

4.	Facility Sanitation

Sanitation of retail and foodservice facilities shall be in compliance with the current edition of the pertinent federal, state, or local Food Code.

5.	Employee Health and Hygiene

Employee health and hygiene policies and practices at retail and foodservice facilities shall be in compliance with the current edition of the pertinent federal, state, or local Food Code.

6.	Preparation within Foodservice/Retail Establishments

a.	Facility

i.	A facility preparing onions shall be designed consistent with the current 				edition of the Food Code and appropriate state and local regulations, 					including but not limited to:

1.)	Floors, walls, and ceilings that can be effectively cleaned and sanitized.

2.)	Closing external doors and windows.

3.)	Water that is adequate and suitable for product and product contact 					surfaces.

4.)	Sufficient hot water for intended use.

5.)	Adequate storing of cleaning and sanitizing chemicals and supplies to 					prevent cross contamination.

6.)	Adequate hand-wash facilities.

7.)	Adequate provisions to wash, sanitize, and dry equipment and utensils.

8.)	Maintain an effective pest control program with no signs of insects or 					rodent activity.

b.	Equipment

i.	When preparing or further handling onions at retail, follow the Food 					Code or state/local requirements regarding facilities and equipment, 					temperature control, cleaning and sanitizing, and personal hygiene.

ii.	Equipment and utensils used to hold, cut, dice, or slice onions should be 					designed for that purpose.  Equipment shall be easily cleaned, free from 					damage that prevents proper cleaning, and stored in a manner that will 					not contribute to product contamination.  Examples of the equipment 					includes, but are not limited to:

1.)	Cutting board

2.)	Thermometers

3.)	Utensils

4.)	Disposable gloves

5.)	Safety gloves

6.)	Finished product containers

c.	Employees preparing cut onions shall adhere to safe food-handling 					practices as directed by the most current edition of the Food Code.  					Employees shall:

i.	Be adequately trained in safe food-handling procedures.

ii.	Be free from symptoms of diagnosed transmissible diseases as defined 					within the most current edition of the Food Code.

iii.	Implement and practice good hand washing procedures, such as at the 					start of the shift, after breaks, visiting restrooms, sneezing, coughing, 					handling trash or money, or anytime hands become soiled.

iv.	Do not allow food, drink, or tobacco products in the food preparation, 					cleaning, or storage areas except as permitted by the Food Code or 					state/local requirements.

v.	Wear clean uniform and/or outer clothing.

vi.	Minimize bare hand contact with onions to be sold as ready-to-eat. 					Options may include clean and sanitary utensils or disposable gloves.

vii.	Utilize hair and beard nets when appropriate.

viii.	Practice good retail practices and food handling techniques to prevent 					cross contamination.

7.	Gloves

When onions are handled with bare hands, documentation of hand-washing procedures must be made as indicated above. If gloves are utilized, a procedure for glove use must be documented and followed.  The following applies to all food service/retail operators who handle onions.

a.	Disposable Gloves

i.	The use of single-use disposable gloves for hand contact with onions is 				recommended.

ii.	Hands shall be washed before putting on gloves.

iii.	Hand sanitizers may be used, but not as a substitute for proper washing of 				hands.

iv.	Disposable gloves must be changed after meals, smoking, using toilet 				facilities, any process involving handling of materials other than onions or 				when gloves become torn, soiled, or otherwise contaminated.

b.	Reusable Gloves

i.	Reusable gloves are not authorized for hand contact of ready-to-eat onions 				at foodservice/retail operations.  When gloves are utilized, only single-				use, disposable gloves should be worn.

8.	Storing Cut/Sliced/Diced Onions

a.	After cutting, onions shall be chilled to and maintained at <41°F.

b.	Cut onions must be stored in a covered container and above other items that 			may cause contamination.

c.	Cut onions that are held longer than 24 hours must indicate the date or day by 			which the food shall be consumed on the premises, sold, or discarded.

9.	Repackaged Dry Bulb Onions

Onions must be stored off the floor and in a manner to prevent cross contamination from raw food products, or unsanitary conditions.

10.	Displaying Cut Onions for the End Consumer

a.	Maintain cut onions at <41°F during display.

b.	If time only is used as a public health control, and allowed by your licensing, 			regulatory authority, written procedures shall be prepared in advance, 				maintained in the food establishment, and made available to the regulatory 			authority upon request. Refer to the current edition of the Food Code for details 			of displaying cut/sliced/diced onions without temperature control.

c.	Packaged cut onions may not be stored in direct contact with ice or water if the 			food is subject to the entry of water because of the nature of its packaging, 			wrapping, or container, or its positioning in the ice or water.

11.	Displaying Dry Bulb Onions for the End Consumer

Dry bulb onions should be clean and free of obvious signs of decay.

12.	Traceability and Record Keeping

a.	All levels of the onion supply chain shall maintain traceability consistent with 			record keeping requirements in 21 CFR part 1, subpart J (1.326-1.368). 				Distributors to direct-to-consumer retail and foodservice operations shall 				maintain traceability to a minimum of one step back (immediate previous 			supplier) and one step forward (immediate next recipient).  Direct-to-consumer 			retail and foodservice operations shall maintain purchase records that will 			facilitate traceability.

b.	Each facility’s ability to comply with the above (12. a) shall be verified at least 			annually. A record of this verification shall be kept on file.

c.	All records recommended in this section shall be maintained for at least six 			months and be readily available.

d.	Recognizing that bulk onions may be commingled in a display, in the event of a 			recall, all lots of commingled lot are affected.

X.	Appendix

●	A Notice to Growers, Food Manufacturers, Food Warehouse Manager, and Transporters of Food Products on How to Dispose of Contaminated Food. Updated September 7, 2006. http://www.cfsan.fda.gov/~dms/fsdisas3.html

●	A Notice to Growers, Food Manufacturers, Food Warehouse Manager, and Transporters of Food Products About the Safety of Food Affected by Hurricanes, Flooding, and Power Outages. Updated September 7, 2006. http://www.cfsan.fda.gov/~dms/fsdisas1.html

●	Guide to Minimize Microbial Food Safety Hazards for Fresh-cut Fruits and Vegetables.2008. http://www,cfsan.fda.gov/~dms/guidance.html

●	Guide to Minimize Microbial Food Safety Hazards for Fresh Fruits and Vegetables.1998. http://www,cfsan.fda.gov/~acrobat/prodguid.pdf

●	Code of Federal Regulations. http://www.gpoaccess.gov/cfr/index.html

●	Model Code for Produce Safety, The Association of Food and Drug Officials, Model Code for Produce Safety for State and Regulatory Agencies (AFDO), November 2009. www.afdo.org/Resources/ProduceSafety.cfm

●	FDA: Federal Code of Regulations. http://www.accessdata.fda.gov/scripts/cdrh/cfdocs/cfcfr/cfrsearch.cfm